*** Installing FUJITSU M2263SA/SB SCSI Disk as NeXT Boot Disk

[Version 2.1, May 20, 1991]

Izumi Ohzawa, izumi@violet.berkeley.edu (standard mail)

izumi@pinoko.berkeley.edu (NeXTmail)

This note was originally posted to a USENET newsgroup comp.sys.next

sometime during the summer 1990.

Updates have been made to include notes for OS Release 2.0/2.1.

With the exception of disk capacity related numbers, procedures described

below should also apply to M2261SA/SB, and M2266SA/SB (1.2GB) drives.

Short descriptions are presented first for using the drive as

(1) internal boot disk in a NeXTcube, and (2) external boot disk for

NeXTstation/cubes.  Long descriptions follow these.

==========================================================================

(1) *** Installing M2263Sx as Internal Boot Disk for NeXTcube.

[For Release 2.0/2.1]

I assume here that the cube can boot from the optical disk drive, or from

ethernet.  If your cube cannot boot from either of these, you will have

to use another NeXT machine to make your new disk bootable.

[I1]  Set the SCSI ID (target#) of the new drive to 1 on CN9.

Remove jumpers at both ends of CNH2 (see step [1]).

For internal disk, the SCSI terminator should be left installed.

(20-pin DIP resister module next to the SCSI connector on the drive)

[I2]  Install the drive in the NeXTcube above the optical drive.

[I3]  Power up the NeXT machine, boot from OD or Ethernet.

[I4]  Login as root.

[I5]  scsimodes /dev/rsd0a

should show the drive parameters of the Fujitsu drive

(see step [3] below).

[I6]  If you feel adventurous, do low-level formatting using 'formatter'.

You generally do not have to perform any low-level format.

Choose the correct SCSI ID, i.e., 1, and press format button.

(or use NeXT-supplied sdform program: /usr/etc/sdform /dev/rsd0a).

(See step [3.5] below for details).

[I7]  From Terminal, do

/usr/etc/builddisk sd0

Wait upto an hour for full disk build.

[I8]  Install boot block.

/usr/etc/disk -b /dev/rsd0a

[I9]  Mount the newly built disk.  Edit etc/fstab on the new disk

to remove/comment out the line for /dev/sd0b (client partition).

Unmount the disk.

[I10] Reboot from the new scsi disk, e.g., in Terminal,

halt

bsd

==========================================================================

(2) *** Installing M2263Sx as External Boot Disk for NeXTstaion/cube.

[For Release 2.0/2.1]

** Warning **

I have not tested this procedure.  It should work, but no guarantees.

Assumption here is that you have a friend who has a 2.0/2.1 Extended

system, and she will let you connect your new drive to her machine

and load the system onto your drive.  In this case, you will be making

the new external drive the boot disk of your system.  The internal disk

that came with the computer will become a secondary disk.  You should

still be able to use the machine without the external disk, if you keep

the minimum configuration intact.

If your machine boots from the internal drive, and you are just adding

a non-boot disk externally, you don't probably have to do anything

under 2.0/2.1 other than setting the jumpers (step [E1] below).

[E1]  Set the SCSI ID (target#) of the new external drive to 2 on CN9.

Remove jumpers at both ends of CNH2 (see step [1]).

If this disk is the only or last device on the external SCSI bus,

leave the terminator ON (20-pin DIP resister module next to the

SCSI connector on the drive).  If the disk is in the middle of

the SCSI chain, remove the terminator.

[E2]  Connect the drive to SCSI port of the main unit (with powers off).

[E3]  Power up the external drive, and then the NeXT machine.

[E4]  Login as root.

[E5]  scsimodes /dev/rsd1a

This should show the drive parameters of the Fujitsu drive.

(see step [3] below).  Make DAMN sure that you specify /dev/rsd1a,

and that you get "FUJITSU ..." message in the output.

Otherwise, you might wipe out your friend's disk accidentally.

[E6]  If you feel adventurous, do low-level formatting using 'formatter'.

You generally do not have to perform any low-level format.

Choose the correct SCSI ID, i.e., 2, and press format button.

(or use NeXT-supplied sdform program: /usr/etc/sdform /dev/rsd1a).

(See step [3.5] below for details).

[E7]  From terminal, do

/usr/etc/builddisk sd1

Wait upto an hour for full disk build.

[E8]  Install boot block.

/usr/etc/disk -b /dev/rsd1a

[I9]  Mount the newly built disk.  Edit etc/fstab on the new disk

to remove/comment out the line for /dev/sd0b (client partition).

Unmount the disk and power down the NeXT and the external disk.

[E10]  Change the SCSI ID (target#) of the drive to 0.  Take the drive home,

and connect to your NeXT.

Your NeXT should now boot from the new external drive.

(The external drive is now /dev/sd0a, and the internal /dev/sd1a.)

[E11] Edit /etc/fstab to add a line to mount the internal drive.

/dev/sd1a /Internal 4.3 rw,noquota 0 2

Of course, you can replace "Internal" with any name you desire as

the directory at which the internal drive is mounted.

[E12] Reboot.

[E13] Clean-up the internal drive, if you want to make space by deleting

non-essential stuff that is duplicated on the external drive.

==========================================================================

This is the long version of the installation note.  Descriptions

primarily apply to the case where the drive is mounted internally

in the NeXTcube.  Please make necessary corrections or disregard

irrelevant descriptions if your drive is externally mounted, e.g.,

changing rsd0a to rsd1a where necessary.

[0] I assume you know how to handle electro-static sensitive devices.

If you don't, read Appendix C "Opening the Cube" of User's

Reference which you should have received with your cube.

[1] Jumper settings on the drive

(a) SCSI target# can be set by jumber block "CN9" located right

next to the SCSI bus connector of the drive.  Three jumpers

closest to the SCSI connector encode the target number in

binary.

View from the back of the drive, with the drive UP-SIDE DOWN.

|-----------|  |----------------------------|       .......

| power con.|  |   50 pin SCSI connector    |   CN9 .......

|-----------|  |----------------------------|       \-/

ID

For SCSI target# 0, remove the three left-most jumpers on CN9.

For SCSI target# 1, keep the left-most one, and remove the

second and third jumper from left.

Figure out other target ID's yourself using the left-most jumber

as LSB (least significant bit).

The recommended target# for internal boot disk is 1.  This

allows an external SCSI drive with target #0 to become the boot

device when such a device is connected (without opening the cube).

(The cube will boot from the SCSI device with the smallest

target #.  Manual booting from the ROM monitor will always allow

booting from a SCSI device with any target #.)

(b) Remove two jumpers at both ends of the jumper block "CNH2".

Locate the jumper block "CNH2" betweeen two surface-mount VSLI's

whose labels are obscured by capacitors ( 1cm square blocks

colored blue on my drive, with two leads).

There are 8 jumpers in this block.

On Jumper Block "CNH2" remove:

[1-2]      INQUIRY DATA

[15-16]    Synchronous Mode Transfer

If you can't figure out the numbers on the block,

[1-2] is at one end and indicated by a little triangle mark, and

[15-16] is at the other end (8-th position from the triangle).

Both of the jumbers in CNH2 have to be pulled out for 68030

(old cube), 68040 CPU boards, and NeXTstation.

(See attached comp.sys.next article at the end.)

[  This information was kindly provided by Helene Young-Myers,

helene@secd.cs.umd.edu, and her husband Eugene D. Myers. ]

You will be able to perform BuildDisk with these two jumpers on,

and will be able to mount and use the drive if it is not used as

the boot disk.

However, the cube will not boot when you try to boot from the

new SCSI drive later in step [8].

These two jumpers are probably the only ones you have to

change from the factory configuration.

On my drive, all other jumpers in block "CNH2" are installed

except for the two above.  All jumpers in block "CNH1" and

"CNH4" are also on.

(c) SCSI Terminator

If you are installing the drive inside the cube, leave the

terminator on (20-pin DIP resister module next to the

SCSI connector on the drive).

If this disk is the only or last device on the external SCSI bus,

leave the terminator ON.  If the disk is in the middle of

the SCSI chain, remove the terminator.

[2] Remove the 40MB swap disk if present, and install M2263S.

Connect the power cable to the drive, and SCSI flat cable.

Make sure that pins (50 each on the SCSI drive, and the

CPU board) are not bent.  Straighten them if any are bent.

If you are using an internal SCSI cable from a third-party

vendor, use caution not to insert the connector upside-down.

The NeXT supplied internal SCSI cable is keyed, so it shouldn't

be possible to insert it upside-down.

If, in step [3], "scsimodes" does not give you anything other

than error messages, you might have connected the cable in the

wrong way.  There is a little label on the connector of the

drive indicating which pin is pin-1.  Make sure that the same

side of the cable goes to pin-1 on the drive and the CPU board.

[3] Boot from OD, and login as user "root".

Use command "scsimodes" as below to get basic formatting

information, e.g., bytes per sector, etc.

If "scsimodes" does not work, the most likely cause is

improper cable connections.

---

For M2263SA

localhost# scsimodes /dev/rsd0a		(/dev/rsd1a if external)

SCSI information for /dev/rsd0a

Drive type: FUJITSU M2263S-512

512 bytes per sector

53 sectors per track

15 tracks per cylinder

1658 cylinder per volume (including spare cylinders)

3 spare sectors per cylinder

15 alternate tracks per volume

1312343 usable sectors on volume

---

For M2263SB (or SA after running "formatter")

localhost# scsimodes /dev/rsd0a		(/dev/rsd1a if external)

SCSI information for /dev/rsd0a

Drive type: FUJITSU M2263S-1024

1024 bytes per sector

28 sectors per track

15 tracks per cylinder

1658 cylinder per volume (including spare cylinders)

3 spare sectors per cylinder

15 alternate tracks per volume

690968 usable sectors on volume

[3.5] Low-level Formatting (*** OPTIONAL ***)

If you have a program "formatter" by Rory Bolt (see below),

you can optionally perform a low-level format of M2263SA.

You can also use a NeXT supplied program /usr/etc/sdform for

this purpose although it cannot do many of the things "formatter" can.

Rory's program has been written specifically for Fujitsu M226xS

series drives although it works with many other SCSI drives.

Your drive should work without this step, however.

Therefore, skip it if you are unconfortable doing it.

Low-level formatting is not really required because the drive comes

pre-formatted from factory.

However, running "formatter" will give you additional 35MB of storage

from M2263SA by essentially turning it into an M2263SB, speeds up the

drive slightly, and it will enable automatic error recovery feature

of the Fujitsu drive.

[You get about 632MB for M2263SA, and 667MB for M2263SB after

i-nodes and other Unix overhead are subtracted.  The difference between

SA and SB is that SA has 512 bytes/sector block size while SB has

1024 bytes/sector.  If you can get M2263SB instead of SA from a

vendor, by all means get the SB version.]

Also, remember for future reference that there are certain SCSI disk

errors which can usually be fixed by low-level formatting a drive.

If your drive, after some use, develops "MEDIA ERRORs" which

will be recorded in the system log file /usr/adm/messages, you might be

able to fix the drive by doing a low-level format.  These media errors

cannot be fixed by simply re-doing BuildDisk and reloading the system.

If a low-level format doesn't revive the disk, it's time to ship the

disk back to the manufacturer.

Low-level formatting is different from the intialization process

performed by BuildDisk or by command line "disk -i /dev/rsd0a".

The init process done by BuildDisk or disk program merely lays

down Unix filesystem(s), while a low-level format freshens formatting and

bad-sector substitution table internal to the SCSI disk.  The low-level

formatting process will take 20-30 minutes for 670MB drive, while the

BuildDisk init process (or "disk -i /dev/rsd0a") will take only a few

minutes.

**** How to get "formatter" (v1.2) by Rory Bolt.

This program is available from the standard NeXT archive sites via

anonymous FTP if you are connected to Internet.  If you do not

have FTP access, Purdue University archive also operates an E-mail

based file server.  This is a shareware program ($15), so please

send a check to Rory if the program works for you.  (Once is

all you need to run this program, usually.  It is also a better

deal than the other program TheFormatter for Wren's.)

[I have no connection with Rory Bolt except as a happy user of his

program.]

The current version (1.2) was develped on NeXT OS 1.0, but works fine

on Release 2.0 too [I've done it.]

NeXT FTP archives are as follows:

Host: nova.cc.purdue.edu or sonata.cc.purdue.edu.

Path: /pub/next/1.0-release/binaries/formatter_1.2.tar.Z (binary file)

Host: cs.orst.edu

Path: /pub/next/binaries/formatter_1.2.tar.Z (binary file)

To obtain formatter_1.2.tar.Z via E-mail based file server:

Send mail to: archive-server@cc.purdue.edu

with the subject line,

Subject: send next/1.0-release/binaries formatter_1.2.tar.Z

[                                      ^-- Not a typo. SPACE here.]

Body of the message can be empty.  You will receive the file

uuencoded.  Size of the program after uuencoding is about 34kbytes,

thus there shouldn't be any problem for it to go through various

gateways.  If you have trouble with the e-mail server,

or want to obtain further information on the Purdue e-mail archive

server, send mail to the same address with,

Subject: help

If you cannot obtain "formatter" from these sites, you can still

perform low-level format with a NeXT supplied program "sdform".

However, "sdform" cannot change the block size from 512 to

1024 bytes/sector, and therefore cannot increase the capacity for

M2263SA.  For locking out bad sectors, "sdform" should

be satisfactory.

Use "sdform" as follows (as user "root"):

localhost# /usr/etc/sdform /dev/rsd0a		(/dev/rsd1a if external)

(This also takes 20-30 minutes.  Do not interrupt once started.)

###### Release 2.0/2.1 Note ################################################

Skip steps [4] and [5] if you have OS Release 2.0/2.1, and want to use the

whole disk as one partition.  No modification to /etc/disktab is needed.

Steps [4] and [5] are relevant only if one the following applies:

(a) Your OS release is still 1.0 or 1.0a.

(b) OS is 2.0/2.1, but you need multiple partitions on the disk.

############################################################################

[4] Add the following entry to the end of "/etc/disktab".

[Not requried for OS Release 2.0/2.1]

If the parameters shown by "scsimodes" are different

from those above, you cannot use the entry below.

If you want the modified disktab to appear also in

the SCSI disk's /etc/disktab after BuildDisk, copy

/etc/disktab to /usr/template/client/etc/disktab, too.

( BuildDisk will copy /usr/template/client/etc/* onto a

newly built disk as /private/etc/* )

Be sure to make backup copies of the unmodified disktab

in each directory.  Write permission may have to be set

to edit these files.

----- cut here and append to /etc/disktab -----------------------------------

# FUJITSU M2263SA disktab

# FUJITSU M2263S-512 with 3 spare sectors/cylinder and 512 byte sectors

# localhost# scsimodes /dev/rsd0a

# SCSI information for /dev/rsd0a

# Drive type: FUJITSU M2263S-512

# 512 bytes per sector

# 53 sectors per track

# 15 tracks per cylinder

# 1658 cylinder per volume (including spare cylinders)

# 3 spare sectors per cylinder

# 15 alternate tracks per volume

# 1312343 usable sectors on volume

#

# This disktab entry from: majka@cs.ubc.ca (Marc Majka)

# Organization: UBC Department of Computer Science, Vancouver, B.C., Canada

# Fujitsu M2263S 671MB 5.25" drive with 512 byte sectors

# M2263S - One partition

M2263S-512-ALL|Fujitsu 671MB w/512 byte sectors as 1 partition:\

:ty=fixed_rw_scsi:nc#1658:nt#15:ns#53:ss#1024:rm#3600:\

:fp#160:bp#0:ng#0:gs#0:ga#0:ao#0:\

:os=sdmach:z0#32:z1#96:ro=a:\

:pa#0:sa#655875:ba#8192:fa#1024:ca#32:da#4096:ra#10:oa=time:\

:ia:ta=4.3BSD:

# M2263S - Two partitions

M2263S-512|FUJITSU M2263S-512|Fujitsu 671MB w/512 byte sectors:\

:ty=fixed_rw_scsi:nc#1658:nt#15:ns#53:ss#1024:rm#3600:\

:fp#160:bp#0:ng#0:gs#0:ga#0:ao#0:\

:os=sdmach:z0#32:z1#96:ro=a:\

:pa#0:sa#250425:ba#8192:fa#1024:ca#32:da#4096:ra#10:oa=time:\

:ia:ta=4.3BSD:\

:pb#250425:sb#405450:bb#8192:fb#1024:cb#32:db#8192:rb#10:ob=time:\

:ib:tb=4.3BSD:

#

# FUJITSU M2263SB disktab, or for M2263Sx after running "formatter".

# localhost# scsimodes /dev/rsd0a

# SCSI information for /dev/rsd0a

# Drive type: FUJITSU M2263S-1024

# 1024 bytes per sector

# 28 sectors per track

# 15 tracks per cylinder

# 1658 cylinder per volume (including spare cylinders)

# 3 spare sectors per cylinder

# 15 alternate tracks per volume

# 690968 usable sectors on volume

#

# Generated by "formatter v1.2" by Rory Bolt after low-level formatting.

# FUJITSU M2263S-1024

M2263S-1024|FUJITSU M2263S-1024|FUJITSU M2263S-1024 w/1024 byte sectors as 1 partition:\

:ty=fixed_rw_scsi:nc#1658:nt#15:ns#28:ss#1024:rm#3600:\

:fp#160:bp#0:ng#0:gs#0:ga#0:ao#0:\

:os=sdmach:z0#32:z1#96:r0=a:\

:pa#0:sa#690809:ba#8192:fa#1024:ca#32:da#4096:ra#10:oa=time:\

:ia:ta=4.3BSD:

#

------ cut here --- end of disktab for M2263S --------------------------------

[5] Check if the modified /etc/disktab is valid by;

[Not requried for OS Release 2.0/2.1]

localhost# disk /dev/rsd0a

disk name: FUJITSU M2263S-512 (or -1024)

disk type: fixed_rw_scsi

Disk utility

disk> quit		(quit this program)

localhost#

If the "disk" command does not recognize the drive, you have

made a mistake in adding the disktab entry in step [4].

Check /etc/disktab again.

[6] Launch "BuildDisk", select HARD DISK and run it.

This takes upto one hour.  If, for some reason, BuildDisk

app does not work, try the command line version from Terminal.

[This happened to me once.  Since then, I am using the

command line version below exclusively.]

localhost# builddisk sd0		(sd1 if external)

This command is a shell script.  Look at /usr/etc/builddisk

if you want to find out exactly what it does.  You can also give

a customized /etc/BLD.* script as an option to this command.

[6.5] Install boot block to the disk.

localhost# disk -b /dev/rsd0a		(/dev/rsd1a if external)

Apparently, BuildDisk (or command line builddisk) does not write

the 2.0 boot block to the disk.  The boot block is needed to load

the Mach kernel into memory as a first step in booting the machine.

The boot block does not exist if you have done a low-level format, or

if you have just received the drive from a dealer.

[7] If you are using the first disktab entry (the whole disk

as one large partition) or if you didn't put a disktab entry

because your OS is 2.0/2.1, you must edit "/etc/fstab" on the

SCSI disk before you attempt to boot from it.

After builddisk, "/etc/fstab" for some reason contains 2

partitions "sd0a", and "sd0b", the latter of which of course

does not exist.  Because of this, booting will fail and the

system will come up in single-user mode in a dumb console

window.  If you are using the second entry (for two partions),

you don't have to modify /etc/fstab.  So, skip this step.

mount /dev/sd0a /mnt

chmod 644 /mnt/etc/fstab

Using your favorite text editor,

edit /mnt/etc/fstab, and comment out or remove the line:

/dev/sd0b /clients 4.3 rw,noquota 0 2

Save the file, and

umount /dev/sd0a

[8] Shutdown and boot from the SCSI disk.

Type 'halt', and you will be left in the ROM Monitor.

Type 'bsd' at the ROM Monitor prompt.

If the ROM monitor complains:

SCSI unexpeced msg:1

sc: Unexpected msg

.... repeated many times...

Then, you must have not set the jumpers correctly in step [1](b).

Once you confirm that the new disk is working, change the default

boot device to hard disk using Preferences app (Power button panel).

===========================================================================

Additional information of interest

[9] Transfer Rate

(This was done with 68030 cube under OS Release 1.0a. BEFORE low-level

format by Rory Bolt's "formatter.")

localhost# disk /dev/rsd0a

disk name: FUJITSU M2263S-512

disk type: fixed_rw_scsi

Disk utility

disk> read

starting block? 0

# sectors per transfer? 16

number of transfers? 1000

sector increment? 16

16384000 bytes in 21243 ms = 771374 bytes/s

disk> read

starting block? 0

# sectors per transfer? 16

number of transfers? 1000

sector increment? 16

16384000 bytes in 21242 ms = 771374 bytes/s

For comparison, according to Ronald Antony & Axel Merk (the authors of

"TheFormatter" for Wren drives), CDC Wren VI (CDC 94191-15) gives

520000 - 560000 bytes/sec for 512 bytes/sector drive without cache, and

900000 - 980000 bytes/sec with on-drive cache.

[10] Seek Noise

The Fujitsu drive is quite a bit noisy during seeks.

It's not as bad as the OD drive as I said in the old note.

You will certainly hear seek noise every time you receive a mail,

when a File Viewer opens a directory or when someone does an rlogin, etc.

By comparison, Maxtor XT-8760S, the NeXT's official 660MB drive,

is dead quiet.  Perhaps, the drive may not be suitable if you need a

really quiet environment.

But the 5-year manufacturer warranty is, for me, quite soothing :-)

I don't know how Fujitsu compares with CDC Wren drives with respect to

seek noise.

[11] As always, use the information above at your own risk.  I am

not responsible for any damage to the SCSI drive or to the

NeXT cube itself (voiding the warranty) caused directly

or indirectly by the use of the information provided above.

==========================================================================

ATTACHMENT

Article 15834 (9 more) in comp.sys.next:

From: ph600fbj@sdcc14.ucsd.edu (james dehnert)

Subject: Sumary: synchronous scsi

Date: 2 Apr 91 00:13:25 GMT

Fisrt of all, thanks to all who responded, especially Mike Perka

at NeXT and Rory Bolt who wrote the SCSI Formatter utility for

Fujitsu M226XS drives (and Common Command Set generic drives as well)

who supplied most all of the information I required.

I am just going to put things the way i got them to avoud

confusion (by me) of what many of us want to know, so..

First from NeXT,

1) The SCSI drivers on NeXT systems do -not- support synchronous

transfers across the SCSI bus.  The drivers do support "synchronous

devices" -- in asynchronous transfer mode.  All SCSI devices that

support synchronous transfers must support asynchronous transfers

as well.  The reverse is not true.

and from  Rory Bolt,

2) Do not attemp to install the synchronous transfer jumper. It will

not damage anything if you do, however, your disk will not be bootable.

Removing the jumper will make your disk bootable again. All this jumper

does is enable the drive to initiate the synchronous transfer negotiation.

Even though the 040 NeXTs are SCSI-II, they do not support targets which

initiate the negotiation.

With the jumper removed the drive still supports synchronous

transfers, but the host must initiate the negotiation (which the NeXT

does). If you insert the jumper the NeXT will complain about unexpected

messages (which are the drive trying to negotiate synchronous transfers).

3) No, you will not have to reformat your drive.

I hope this is enough info for everyone.\

Thanks again to all who responded,

Zeke