Disclaimer

Novell, Inc. makes no representations or warranties with respect to

the contents or use of this manual, and specifically  disclaims any

express or implied warranties of merchantability or fitness for any

particular purpose.  Further, Novell, Inc. reserves the right to

revise this publication and to make changes to its content, at any

time, without obligation to notify any person or entity of such

revisions or changes.

Further, Novell, Inc. makes no representations or warranties with

respect to any NetWare software, and specifically  disclaims any

express or implied warranties of merchantability or fitness for any

particular purpose.  Further, Novell, Inc. reserves the right to

make changes to any and all parts of Netware software, at any time,

without obligation to notify any person or entity of such revisions

or changes.

Novell, Incorporated

122 East 1700 South

Provo, Utah  84606 USA

Trademarks

NetWare and Novell are registered trademarks of Novell, Inc.

IBM is a registered trademark of IBM, Inc.

References

The following publications contain information that may be helpful

in reading this document:

1) NOVELL NetWare Version 3.11 Installation

(See Appendix D: Using DOSGEN to Add Remote Reset)

2) IBM Local Area Network Technical Reference, SC30-3383-3

3) IBM Remote Program Load User's Guide, 83X7840

4) IBM Token-Ring Network Architecture Reference, SC30-3374-02

(c) Copyright 1995 Novell, Inc.  All rights reserved.  No part of

this publication may be reproduced, photocopied, stored on a

retrieval system, or transmitted without the express prior written

consent of the publisher.

RPL.nlm is a NetWare Loadable Module that acts as a Protocol

Stack and responds to the IBM architected Remote Program Load (RPL)

frames as defined in the "IBM Remote Program Load User's Guide".

It is used in networks that have diskless workstations installed

with the RPL BIOS Module.  Currently, this is supported on the

following network adapters:

IBM Ethernet Adapters

IBM PC Network Adapters

IBM Token-Ring Network Adapters

Specifically, RPL.nlm will respond to the following frames:

FIND                RPL.nlm will respond with a FOUND frame.

SEND.FILE.REQUEST   RPL.nlm will respond with a

FILE.DATA.RESPONSE frame.

RPL.nlm is intended to be a replacement for the following NetWare

v3.11 modules:

PCN2LRPL.nlm   For networks using the IBM PC Network Adapter.

ETHERRPL.nlm   For networks using the IBM Ethernet Adapter.

TOKENRPL.nlm   For networks using the IBM Token-Ring Adapter.

Features and Fixes of RPL.nlm

RPL.nlm comes with five new and improved BOOTSTRAP programs:

ETHER.rpl, F1ETH.rpl, PCN2L.rpl, RBOOT.rpl, and TOKEN.rpl.

Together, these files offer the following features and fixes:

BOOTCONF.sys        Wildcard characters (* and ?) are allowed

in specifying Node Addresses.  Also, more

than one Disk Image File name is allowed

per Node Address.  BOOTCONF.sys is parsed

by RPL.nlm at the File Server to minimize

the amount of Network Traffic.

IBM LAN Support Program  RPL now works reliably the the IBM

LAN Support Program.

RPLFIX and RPLODI   These two programs are NOT required when

booting a workstation.

Source Routing      You may now boot across a Source Routing

Bridge.

PS/2 REFERENCE      The *.rpl files will communicate with the

BIOS on a PS/2 machine and ask it if it needs

to run REFERENCE.  If the BIOS says "yes" to

this query, RBOOT.RPL, ETHER.RPL, TOKEN.RPL,

F1ETH.RPL, AND PCN2L.RPL will automatically

present a menu on the workstation screen

with a "MXX.REF" boot image file name, where

"XX" is the PS/2 Model number returned by

the PS/2's BIOS.  This file name is for

the reference diskette image that needs to

be dosgened and copied into the \LOGIN

directory.  An example of one of these file

names would be:  MF8.REF where F8 is the model

number returned by a PS/2 Model 56LS BIOS.

Installing RPL.nlm on the File Server

The following BOOTSTRAP programs come with RPL.nlm and should be

installed in the SYS:\LOGIN directory of the file server:

ETHER.rpl      For IBM MCA Ethernet Adapters

F1ETH.rpl      For IBM Model 25SX Ethernet Adapters

PCN2L.rpl      For IBM PC Network Adapters

RBOOT.rpl      For Adapters using the Novell Boot ROM Kit

TOKEN.rpl      For IBM Token-Ring Network Adapters

An appropriate BOOTSTRAP program gets downloaded to the workstation

in response to a SEND.FILE.REQUEST from the Boot ROM RPL Module.

RPL.nlm should be installed in the SYS:\SYSTEM directory of the

file server.  In addition, certain IBM Micro-Channel computers,

such as the new 386SLC, have a BIOS image file associated with

them.  This file comes with the computer on a diskette, and has a

file extension of .IML.  You MUST create a directory called

SYS:\LOGIN\IBMLAN\DCDB\IMAGES and install ALL .IML files in this

directory.

Loading RPL.nlm on the File Server

RPL.nlm is loaded the same as any NetWare NLM:

LOAD RPL

at the file server command prompt.  There are no parameters

associated with loading RPL.nlm.

Binding RPL.nlm to the 802.2 Board

Since RPL.nlm is a Protocol Stack, it must be bound to any and all

boards that have RPL clients attached to them:

BIND RPL to board [ACK],[FRAME=ff],[GNS],[NODEFAULT],

[PROTECT],[PS=server],[TRO],

[WAIT TIME=ssss]

where board is the name of any NetWare LAN driver that is

configured for IEEE 802.2 frame type.

RPL.nlm BIND Time Parameters

The parameters specified by [..] are OPTIONAL, NOT Case Sensitive,

separated by blanks or commas, and may be entered in any order.

They are described below:

ACK       Use this parameter if you wish to configure the RPL

BOOT ROM Module to ACKnowledge FILE.DATA.RESPONSE

frames sent by RPL.nlm.  By DEFAULT, RPL.nlm will

send FILE.DATA.RESPONSE frames in a BURST mode.

This parameter allows pacing by the workstation, if

the Adapter on the workstation cannot keep up with

RPL.nlm.

FRAME=ff  Use this parameter if you wish to configure the

BOOTSTRAP program to use a frame type other than

the default to access the file server.  Valid

responses are:

FRAME=802.2    This is the DEFAULT.

FRAME=EII      Configure the BOOTSTRAP program for

ETHERNET_II.  This option should

only be used on ETHERNET networks.

FRAME=SNAP     Configure the BOOTSTRAP program for

SNAP.

If FRAME=EII or FRAME=SNAP is selected, RPL.nlm

will force the BOOTSTRAP program to do a Get

Nearest Server (GNS).

The FRAME parameter is fully supported on BOOT ROMs

that use the RBOOT.rpl BOOTSTRAP program.  This

parameter is IGNORED on BOOT ROMs that use

PCN2L.rpl and TOKEN.rpl.  However, FRAME=EII or

FRAME=SNAP will configure ETHER.rpl to use

ETHERNET_II instead of 802.3.  Specifying FRAME=EII

on a Token-Ring BOOT ROM will be ignored.

GNS       This parameter specifies that you wish the

workstation to do a Get Nearest Server request when

the appropriate BOOTSTRAP program gets downloaded.

Normally, RPL.nlm will fill in the bootstrap

program with the file server information, so that

it does NOT need to do a Get Nearest Server

request.  Using this parameter MAY cause the

workstation to find a server OTHER than the one

where RPL.nlm is located.

NODEFAULT This parameter tells RPL.nlm to NOT respond to a

FIND frame UNLESS the node address of the

workstation is found in the BOOTCONF.sys file.  It

is provided for security reasons.  The workstation

WILL NOT boot until the system administrator

inserts into the BOOTCONF.sys file the node address

and associated Disk Image File name(s) to use when

booting the workstation.  A further description of

BOOTCONF.sys is given under the heading of Unique

Boot Sequences using RPL.nlm.

PROTECT   This parameter tells RPL.nlm to configure the

BOOTSTRAP program so that it will protect itself in

the workstation memory.  It does this by adjusting

the Memory Size variable in the BIOS data area

(40:13) to reflect the amount of memory that it

uses.  Using this parameter will REDUCE the amount

of memory that the workstation has available for

DOS by about 12k.  It is recommended that this

parameter NOT be USED unless absolutely necessary.

PS=server This parameter tells the BOOTSTRAP program to

attach to a NetWare file server other than the one

where RPL.nlm is loaded.  server is the name of a

NetWare file server that contains the Image file

for the workstation.

TRO       This parameter specifies that you wish the

BOOTSTRAP program to do a This Ring Only count of

THREE (3) on all Broadcasts frames.  It is useful

in a SOURCE ROUTING environment and Servers are

available on the local ring.

WAIT TIME=ssss This parameter specifies the number of SECONDS

you wish the workstation to wait before

automatically selecting the cursored Disk

Image Name when multiple Disk Image Names are

specified in BOOTCONF.sys.  (See "BOOTCONF.sys

Extensions")

The DEFAULT for this parameter is 0000, which

means forever.  Any value from 0000 to 65535

is allowed.

Using RPL.nlm from a DiskLess Workstation

To use RPL.nlm, you must have installed the RPL ROM Module on your

network adapter board in the workstation.  This module must be

capable of sending the IBM architected RPL frame sequence.  See the

"IBM Remote program Load User's Guide" for information on this

architecture.

Implementing the RPL function consists of creating a Disk Image

File in the SYS:\LOGIN directory of the file server. A description

of this process is given in the NOVELL NetWare Version 3.11

Installation manual.

At the server console you must load and bind RPL.nlm:

LOAD RPL

BIND RPL to board [ACK],[FRAME=ff],[GNS],[NODEFAULT],

[PROTECT],[PS=server],[TRO]

where board is any board that is configured for IEEE 802.2 frame

type.  When you boot the workstation, the Disk Image File you

created with DOSGEN will get executed.

Unique Boot Sequences using RPL.nlm

BOOTCONF.sys is an ASCII text file that allows you to specify a

unique Disk Image File for each workstation that needs access to

different files.  You create BOOTCONF.sys using your favorite text

editor.  The process of creating unique Disk Image Files is

described in detail in the NOVELL NetWare Version 3.11 Installation

manual.

BOOTCONF.sys Extensions

When RPL.nlm loads it will search the SYS:\LOGIN directory of the

file server for a BOOTCONF.sys file.  If it finds it, it will read

it into a memory buffer so that it can parse it when a FIND frame

is received from a workstation.  Note that the parsing of

BOOTCONF.sys is done by RPL.nlm and NOT the BOOTSTRAP program to

minimize the amount of traffic on the network during the RPL

process.  The extensions to BOOTCONF.sys are given in this section.

Using Wildcard Characters in BOOTCONF.sys

Wildcard characters (* and ?) are allowed in the line specifying

the node address of the workstation.  This will allow the system

administrator more flexibility in building the BOOTCONF.sys file.

The rules for these wildcard characters are:

*    Use the ASTERISK character to specify a RANGE of digits

in the node address. For example, if the node address of

the workstation is 10005A123456, it may be specified as

0x*123456 in BOOTCONF.sys.  In this example, RPL.nlm will

match the node address with ANY node address that ends in

123456.

Note that only ONE asterisk (*) may appear in the node

address.

?    Use the QUESTION MARK character to specify ANY digit in

the node address.  In the example above, the node address

could be specified as 0x??????123456 which is equivalent

to 0x*123456.

You may use wildcard characters to specify a DEFAULT Disk

Image File for all workstations on the network that do

not have a specific Disk Image File.  You do this by

placing the line:

0x* = DEFAULT.sys

or        0x???????????? = DEFAULT.sys

as the LAST line in BOOTCONF.sys.  Either one of these

lines will match on ALL workstation node addresses.  The

DEFAULT.sys (or any name you desire) Disk Image File is

generated by DOSGEN, the same as any Disk Image File.

More than ONE Disk Image File per Node Address

Each line in BOOTCONF.sys that contains a node address may specify

more than one Disk Image File name, separated by one or more BLANK

characters.  In this case, the BOOTSTRAP program will present the

workstation user a PROMPT at boot up time to select the Disk Image

File to boot.  For example,  if a workstation's node address is

10005A123456, the line:

0x10005a123456 = ONE.sys TWO.sys THREE.dos

will cause the BOOTSTRAP program to present:

Place CURSOR on DISK IMAGE file; Hit ENTER when Ready:

ONE.sys

TWO.sys

THREE.dos

on the workstation screen.  The BOOTSTRAP program will then

use NCP calls to open the selected Disk Image File.  If it

does NOT exist, it will display:

Unable to OPEN Disk Image File

and retry the operation.

Up to TEN Disk Image File names may be entered for each node

address in BOOTCONF.sys.  Note that they must be separated by

one or more BLANK characters, and they must all fit on ONE

line.

Multiple Lines per Node Address

The ASCII COLON (:) can be used to allow for multiple lines when

specifying a particular node address in BOOTCONF.sys.  It is

provided for convenience when specifying multiple parameters on the

Node Address line.

To use this feature, place the ASCII COLON (:) at the end of the

line.  Note that it must be preceded by at LEAST one ASCII blank:

0x10005A460025 = NET$DOS.sys FRED.sys :

JOE.sys

BOOTCONF.sys BIND Override Parameters

When RPL.nlm parses BOOTCONF.sys, it allows the user the override

the BIND time parameters with parameters specific to a particular

workstation that is being booted.  By default, the parameters that

were entered at BIND time apply to ALL workstations that are

attached to the particular board specified in the BIND command.

The following commands are allowed on a per NODE basis, which, if

used, will override the BIND time parameters:

ACK       Use this parameter if you wish to configure the RPL

BOOT ROM Module to ACKnowledge FILE.DATA.RESPONSE

frames sent by RPL.nlm.  By DEFAULT, RPL.nlm will

send FILE.DATA.RESPONSE frames in a BURST mode.

This parameter allows pacing by the workstation, if

the Adapter on the workstation cannot keep up with

RPL.nlm.

FRAME=ff  Use this parameter if you wish to configure the

BOOTSTRAP program to use a frame type other than

the default to access the file server.  Valid

responses are:

FRAME=802.2    This is the DEFAULT.

FRAME=EII      Configure the BOOTSTRAP program for

ETHERNET_II.  This option should

only be used on ETHERNET networks.

FRAME=SNAP     Configure the BOOTSTRAP program for

SNAP.

If FRAME=EII or FRAME=SNAP is selected, RPL.nlm

will force the BOOTSTRAP program to do a Get

Nearest Server (GNS).

The FRAME parameter is fully supported on BOOT ROMs

that use the RBOOT.rpl BOOTSTRAP program.  This

parameter is IGNORED on BOOT ROMs that use

PCN2L.rpl and TOKEN.rpl.  However, FRAME=EII or

FRAME=SNAP will configure ETHER.rpl to use

ETHERNET_II instead of 802.3.  Specifying FRAME=EII

on a Token-Ring BOOT ROM will be ignored.

GNS       This parameter specifies that you wish the

workstation to do a Get Nearest Server request when

the appropriate BOOTSTRAP program gets downloaded.

Normally, RPL.nlm will fill in the bootstrap

program with the file server information, so that

it does NOT need to do a Get Nearest Server

request.  Using this parameter MAY cause the

workstation to find a server OTHER than the one

where RPL.nlm is located.

NOACK     This parameter will override the ACK parameter

specified on the BIND command.

NOGNS     This parameter will override the GNS parameter

specified on the BIND command.

NOPROTECT This parameter will override the PROTECT parameter

specified on the BIND command.

NOTRO     This parameter will OVERRIDE the TRO parameter if

specified on the BIND command.

PROTECT   This parameter tells RPL.nlm to configure the

BOOTSTRAP program so that it will protect itself in

the workstation memory.  It does this by adjusting

the Memory Size variable in the BIOS data area

(40:13) to reflect the amount of memory that it

uses.  Using this parameter will REDUCE the amount

of memory that the workstation has available for

DOS by about 12k.  It is recommended that this

parameter NOT be USED unless absolutely necessary.

PS=server This parameter tells the BOOTSTRAP program to

attach to a NetWare file server other than the one

where RPL.nlm is loaded.  server is the name of a

NetWare file server that contains the Image file

for the workstation.

REP       string1|string2 allows you to REPlace ALL

occurrences of string1 with string2 in the Disk

Image File.  The '|' (ASCII 7Ch) delimiter is

REQUIRED to delimit the string values.

Use this parameter to DYNAMICALLY re-configure a

Disk Image File during the RPL process.  It is

useful for tailoring files such as AUTOEXEC.BAT or

CONFIG.SYS to a specific workstation.

The rules for using REP are given below:

1)   The search IS case sensitive.  The BOOTSTRAP

program will search for string1 EXACTLY as it

is entered in BOOTCONF.sys.

2)   ALL occurrences of string1 will be replaced

with string2 in the Disk Image File.

3)   string2 MUST be equal to OR shorter than

string1.

4)   If string2 is shorter than string1, the Disk

Image File will be padded with ASCII Blanks

when the substitution is made.

5)   string2 must contain NO embedded ASCII Blanks.

The FIRST blank that is encountered is

interpreted as the end of the string.

TRO       This parameter specifies that you wish the

BOOTSTRAP program to do a This Ring Only count of

THREE (3) on all Broadcasts frames.  It is useful

in a SOURCE ROUTING environment and Servers are

available on the local ring.

WAIT TIME=ssss This parameter specifies the number of SECONDS

you wish the workstation to wait before

automatically selecting the cursored Disk

Image Name when multiple Disk Image Names are

specified in BOOTCONF.sys.

The DEFAULT for this parameter is 0000, which

means forever.  Any value from 0000 to 65535

is allowed.

RPL.nlm parses the Node Address line of BOOTCONF.sys looking for

these keywords.  If an entry if found, but does not match one of

the keywords, it is assumed to be a Disk Image File name.

Therefore, you should NOT have a Disk Image File named the same as

any of these keywords.  Note that these parameters are OPTIONAL,

NOT Case Sensitive, separated by blanks, and may be entered in any

order.

An example of a BOOTCONF.sys line using these parameters is:

0x*1234 = NET$DOS.sys gns protect rep NODE=^^^^^|NODE=67890

In this case, gns and protect will be interpreted as KEYWORDS and

not Disk Image File names. In addition, the string 'NODE=^^^^^'

will be REPLACED with 'NODE=67890' wherever it occurs in the Disk

Image File.

Changing BOOTCONF.sys

RPL.nlm reads BOOTCONF.sys into a memory buffer at LOAD time.  If

the user changes it AFTER BOOTCONF.sys is loaded, RPL.nlm will

detect the change and re-load it automatically.  There may be a

five second delay from the time the changes are saved and the time

RPL.nlm invokes the changes.

RPL.nlm will suspend processing of frames while BOOTCONF.sys is

being loaded into memory.