Subject: When & how to use RPLODI.COM

*** RPLODI.COM is recommended only for Novell IPX Remote Boot Proms

and only when loading LSL.COM, NE2000.COM (OR OTHER MLID),

IPXODI.COM, and NETX or VLMS from the boot image file.  Reason:

Novell IPX Remote Boot proms are burned-in with dedicated IPX

code and expects dedicated IPX drivers to load from NET$DOS.sys.

Instead, when ODI drivers load from the NET$DOS.sys the boot

prom code cannot communicate to the ODI modules as they begin

to load and take control.  Therefore, RPLODI.com provides the

needed code to allow the dedicated IPX boot prom code to

effectively communicate with the ODI drivers while they load.

*** RPLODI.COM is not recommended for Novell Enhanced Boot (RPL) Proms

or with IBM RPL Proms even when loading ODI drivers at the client.

HOW TO DETERMINE THE TYPE OF PROM YOU HAVE:

NOVELL IPX REMOTE BOOT PROM:  At initialization, the Novell IPX Remote

Boot Prom sends an IPX protocol "GET NEAREST SERVER" frame onto the wire.

This "GET NEAREST SERVER" frame is positive identification that this

prom is a Novell IPX Remote Boot Prom.  You can now use RPLODI.com.

NOVELL ENHANCED REMOTE BOOT PROM:  At initialization, the Novell

Enhanced Remote Boot Prom sends an 802.2 protocol "FIND" frame onto

the wire.  This "FIND" frame is positive identification that this

prom is a Novell Enhance Remote Boot Prom that uses the "FIND-FOUND"

802.2 protocol to attach to a server and download a bootstrap file.

Do NOT use RPLODI.com with this type of prom.  RBOOT.RPL already

has the ability to deal with ODI drivers loading from boot images.

IBM RPL PROM:  At initialization the IBM RPL Prom send an 802.2

protocol "FIND" frame onto the wire.  This "FIND" frame is positive

identification that this prom uses the "FIND-FOUND" 802.2 protocol

to attach to a server and download a bootstrap file.  Do NOT use

RPLODI.com with this IBM RPL Prom.   RPLODI.com's intelligence has

already been built into the following files: TOKEN.RPL, ETHER.RPL,

PCN2L.RPL, F1ETH.RPL, RBOOT.RPL.

For Novell IPX Remote Boot Proms, rplodi.com is implemented as shown in the

following AUTOEXEC.BAT -- the load order is very important -- RPLODI.COM

must load immediately after LSL.COM:

AUTOEXEC.BAT

LSL

RPLODI

NE2000

IPXODI

NETX

The above autoexec.bat must be dosgened into the NET$DOS.SYS file.